Firewall

- Open the packet tracer.

- Add 3 laptops, switch, router and server.

- Link all the devices together.

- Chang the Gateway for all laptops to : 192.168.1.1

- Change the following setting for each laptop:

Laptop1:

IP address 192.168.1.11

Subnet mask 255.255.255.0

Laptop2:

IP address 192.168.1.12

Subnet mask 255.255.255.0

Laptop3:

IP address 192.168.1.13

Subnet mask 255.255.255.0

- In the router change the IP configuration for each interface:

Interface 0:

IP address 192.168.1.1

Subnet mask 255.255.255.0

Port Status: ON

Interface 1:

IP address 10.10.10.11

Subnet mask 255.0.0.0

Port Status: ON

- In the Server change the following setting:

Gateway 10.10.10.11

IP Address 10.10.10.128

Subnet Mask 255.0.0.0

Port Status: ON

- From laptop 1 :

Ping laptop2 and the server.

- In the Router Command Line Interface write the following command:

access-list 101 deny icmp any any host-unreachable

access-list 101 permit tcp any any eq www

interface fastethernet0/0

ip access-group 101 in

- From laptop 1 :

Ping the server.

- Use the simulation panel to trace the packet movement.