Put a router (Generic ) router –PT

Put a switch ( 2960 )

Put 3 PCs and 3 Servers

Connect them all using cooper straight

1)Go to the router and change the IP configuration :

IP Address: 192.168.1.1

Subnet Mask: 255.255.255.0

Port Status: ON

2) Then go to the server 0 ( name it DHCP Server)

Configure the IP from desktop

IP address: 192.168.1.2

Subnet mask: 255.255.255.0

Default gateway: 192.168.1.1

Then choose from services DHCP

Press on

Gateway: 192.168.1.1

DNS : 192.168.1.4

3) Go to the server 1 ( name it Web Server)

Configure the IP from the desktop

IP address: 192.168.1.3

Subnet mask: 255.255.255.0

Gateway: 192.168.1.1

Go to service and choose HTTP

On the box write

Pnu.com : )

Like below:

4) Then go to server 2 (name it DNS Server)

Configure the IP from the desktop

IP address: 192.168.1.4

Subnet mask: 255.255.255.0

Default gateway: 192.168.1.1

Go to DNS from service

Press on

Type pnu.com in the Name

Type the address 192.168.1.3

Then press add

5) Then go the pc 0 ( name dynamic 0)

Choose DHCP

And also pc 1  (name it dynamic 1) choose DHCP

For PC2 ( static ) configure the IP

IP address: 192.168.1.7

Subnet mask:  255.255.255.0

Gateway : 192.168.1.1

DNS: 192.168.1.4

The prompt :

- Ping (ICMP)

  - From a client computer use the Desktop Command prompt to ping the other client computers and the servers.

  - Example: From the pc 0 or pc 1client, C> ping 192.168.1.2

  - The first one or two pings may fail, but you should receive a reply on the later pings. This is due to the ping timing out while the ARP process takes place (later).

- Web Browser (HTTP)

  - On the client computers use the Desktop Web Browser, enter the URLs of the Web Servers pnu.com

  - You should see the web pages that you created on these servers.